數位系統導論期末範例5

Bubble Sort演算法離散事件建模和電路合成

陳慶瀚

2022-06-08

1. Bubble Sort演算法

C Code

unsigned char rama[10]={1,3,5,7,9,2,4,6,8,0};
unsigned char terapl;
unsigned char terap2,
for(int i=0;i<10;i++)
{
   terapl=rara[i];
   for(int j=i+1;j<10;j++)
   {
      terap2=rara[j];
      if(terapl>terap2)
      {
         rara[i]=terap2;
         rara[j]=terapl;
         terapl=terap2,
      }
   }

}

2. GRAFCET建模

控制器電路合成

grafcet: process(CLK,RST)

begin

if RST='0' then

X0<='1';X1<='0';X2<='0';

X3<='0';X4<='0';X5<='0';

X6<='0';X7<='0';X8<='0';

X9<='0';

elsif CLK'event and CLK='1' then

if X0='1' and START='1'       then X0<='0';X1<='1';

elsif X1='1'                     then X1<='0';X2<='1';

elsif X2='1'                     then X2<='0';X3<='1';

elsif X3='1' and (temp1 > temp2) then X3<='0';X4<='1';

elsif X3='1' and (temp1 <=temp1) then	X3<='0';X5<='1';

elsif X4='1'                     then	X4<='0';X5<='1';

elsif X5='1'                     then	X5<='0';X6<='1';

elsif X6='1' and (j<10)          then	X6<='0';X2<='1';

elsif X6='1' and (j>=10)         then	X6<='0';X7<='1';

elsif X7='1'                     then	X7<='0';X8<='1';

elsif X8='1' and (i<9)           then	X8<='0';X1<='1';

elsif X8='1' and (i>=9)          then	X8<='0';X9<='1';

elsif X9='1' and START='0'       then	X9<='0';X0<='1';

end if;

end if;

end process;

Datapath電路合成

datapath: process(CLK,RST)

begin

if CLK'event and CLK='1' then

if X0='1' then i<=0;FINISH<='0';

elsif X1='1' then temp1<=ram(i);j<=i+1;

elsif X2='1' then temp2<=ram(j);

elsif X4='1' then ram(i)<=temp2;ram(j)<=temp1;temp1<=temp2;

elsif X5='1' then j<=j+1;

elsif X7='1' then i<=i+1;

elsif X9='1' then FINISH<='1';

end if;

end if;

end process;

系統合成

LIBRARY IEEE;

USE IEEE.STD_LOGIC_1164.all;

USE IEEE.STD_LOGIC_ARITH.ALL;

USE IEEE.STD_LOGIC_UNSIGNED.ALL;

entity bubble_sort is

port(

clk,rst,START : in std_logic;

FINISH :out std_logic;

r0,r1,r2,r3,r4,r5,r6,r7,r8,r9	: out integer range 0 to 15

);

end bubble_sort;

architecture arch of bubble_sort is

signal  X0,X1,X2,X3,X4,X5,X6,X7,X8,X9 : std_logic;

signal  temp1,temp2 	: integer range 0 to 15;

signal  i,j 			: integer range 0 to 15;

type    ram_type is array (0 to 9) of integer range 0 to 15;

signal  ram: ram_type :=(1,3,5,7,9,2,4,6,8,0);

begin

grafcet: process(CLK,RST)

begin

……

end process;

datapath: process(CLK,RST)

begin

……

end process;

r0	<= ram(0);

r1	<= ram(1);

r2	<= ram(2);

r3	<= ram(3);

r4	<= ram(4);

r5	<= ram(5);

r6	<= ram(6);

r7	<= ram(7);

r8	<= ram(8);

r9	<= ram(9);

end arch;

波形模擬